| S.N | Name of the Farm Implements with required Technical Specifications |  | Supplier's Specifictions | Rate in figure(Including vat for applicable items) | Rate in word | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Minitiller(Petrol) |  |  |  |  |  |
|  | Brand/Model | ISO Certified Mini tiller ,Model Engine Model Number:…............... |  |  |  |  |
|  | General | General purpose Mini Tiller for dry and wet tillage, with Standard design having gross vehicle weight not less than 75 kg |  |  |  |  |
|  | Engine | Single cylinder,Four Stroke, Inclined/vertical forced air cooled system,oil batch type filter ,Petrol engine ,maximum power not less than 4KW at rated speed |  |  |  |  |
|  | Starting type | Recoil/Electrical Starter |  |  |  |  |
|  | Transmission System | Double Clutch type, Gear drive(1-Reverse, 2-Forward)(Both high and low) number of gears |  |  |  |  |
|  | Displacement (ml(cu.in)) | not less than 208CC |  |  |  |  |
|  | Fuel Tank Capacity | not less than 3L |  |  |  |  |
|  | Blades | minimun 24 |  |  |  |  |
|  | Tilling Depth | 100mm or above |  |  |  |  |
|  | Tilling Width | 750mm or above |  |  |  |  |
|  | Warranty | Minimum 1 year warranty should be provided |  |  |  |  |
| 2 | Mini tiller (diesel) |  |  |  |  |  |
|  | Brand/Model | ISO Certified Mini tiller ,Model Engine Model Number:…............... |  |  |  |  |
|  | General | General purpose Mini Tiller for dry and wet tillage, with Standard design having gross vehicle weight not less than 120 kg |  |  |  |  |
|  | Engine | Single cylinder,Four Stroke, Inclined/vertical forced air cooled system,oil batch type filter ,Diesel engine ,maximum power not less than 4KW at rated speed |  |  |  |  |
|  | Starting type | Recoil/Electrical Starter |  |  |  |  |
|  | Transmission System | Double Clutch type, Gear drive(1-Reverse, 2-Forward)(Both high and low) number of gears |  |  |  |  |
|  | Tilling Depth | 100mm or above |  |  |  |  |
|  | Tilling Width | 750mm or above |  |  |  |  |
|  | Fuel Tank Capacity | minimum 3.5liters |  |  |  |  |
|  | Blades | minimum 32 pieces |  |  |  |  |
|  | Tyre | Pneumatic tubeless tyre |  |  |  |  |
|  | Colour | Standrad |  |  |  |  |
|  | Warranty | Minimum 1 year warranty should be provided |  |  |  |  |
| 3 | Paddy Thresher |  |  |  |  |  |
|  | Make,Model,Brand | Any |  |  |  |  |
|  | General | It is used for threshing paddy,maize ,wheat ,sorghum ,barley,provided at least 3 types of grain sieve |  |  |  |  |
|  | Type | 4WD driven |  |  |  |  |
|  | Power requirement (hp/KW) | above 40HP ,tractor pto |  |  |  |  |
|  | Working efficiency | ≥1200kg/hr |  |  |  |  |
|  | Threshing efficiency | ≥97% |  |  |  |  |
|  | Overall grain loss | ≤2.5% |  |  |  |  |
|  | Broken grain loss | ≤0.8% |  |  |  |  |
|  | Blown grain loss | ≤1.5% |  |  |  |  |
|  | Crop feeding | direct hopper type(feeding chute ,manually) |  |  |  |  |
|  | Type of cylinder | spike tooth |  |  |  |  |
|  | Cylinder size,tip diameter *length (m m) | approx.770*1500 |  |  |  |  |
|  | No. of sieves | 3 |  |  |  |  |
|  | No. of blower | one /double/trible |  |  |  |  |
|  | No. of spike | around 75 |  |  |  |  |
|  | Cleaning fan | at least 1 aspiratory fan for grain clean and 1 centrifugal fan for dust clean |  |  |  |  |
|  | Weight (approx.) | 1450-1550kg |  |  |  |  |
|  | Standard Accessories | PTO shaft ,tool box ,greas gun |  |  |  |  |
|  | Main shaft diameter | not less than 60mm |  |  |  |  |
|  | No. of transmission joint | 4 |  |  |  |  |
|  | Warranty | Minimum 1 year warranty should be provided |  |  |  |  |
| 4 | Combine Mill |  |  |  |  |  |
|  | Make and model | Any |  |  |  |  |
|  | General | Application in both milling of cereals such as rice,wheat,millet and paddy ,Gross wt not less than 85kg |  |  |  |  |
|  | Power source | Not less than 2KW |  |  |  |  |
|  | Motor | 100% copper |  |  |  |  |
|  | Voltage and frequency of motor | 220volt,50Hz |  |  |  |  |
|  | Seive size | at least 100*749mm |  |  |  |  |
|  | Seive diameter | aperature : 1.2 |  |  |  |  |
|  | Capacity | Rice mill: at least 80kg/hr |  |  |  |  |
|  |  | Grain Grinder -at least 130kg/hr |  |  |  |  |
|  |  | Grain Grinder -5500rev/min |  |  |  |  |
|  | Roller Diameter :Rice mill | 40mm |  |  |  |  |
|  | Types | Disc hammer types |  |  |  |  |
|  | Accessories | Electric wire along with suitable MCB with switch |  |  |  |  |
|  | Warranty | Minimum 1 year warranty should be provided |  |  |  |  |
| 5 | Corn Sheller |  |  |  |  |  |
|  | Make & Model | Any |  |  |  |  |
|  | Power | At least 0.65 Kw |  |  |  |  |
|  | Motor | Copper Coil 100% |  |  |  |  |
|  | Capacity | 600-700 kg/hr |  |  |  |  |
|  | Shaft Speed | 1680 r/min |  |  |  |  |
|  | Rotor Diameter | Around 590 mm |  |  |  |  |
|  | Tube | Single |  |  |  |  |
|  | Warranty | Minimum 1 year warranty should be provided |  |  |  |  |
| 6 | Battery cum Manual Sprayer (2 in 1) |  |  |  |  |  |
|  | Tank Capacity | At least 16 liters |  |  |  |  |
|  | Motor | 12V motor powered diaphragm pump of speed 3300 rpm 12V, 8AHr rechargeable, aluminum coated |  |  |  |  |
|  | Working pressure | At least 0.4 Mpa |  |  |  |  |
|  | Charge input | 100-240V AC,50Hz, |  |  |  |  |
|  | Charge output | 12V DC, 1.5A |  |  |  |  |
|  | Spray flow | Not less than 2100ml\min |  |  |  |  |
|  | Time of full charge | 5 to 8 hr |  |  |  |  |
|  | Gross weight | At least 2.6 kg |  |  |  |  |
| 7 | Battery cum Manual Sprayer (2 in 1) |  |  |  |  |  |
|  | Tank Capacity | At least 16 liters |  |  |  |  |
|  | Motor | 12V motor powered diaphragm pump of speed 3300 rpm 12V, 8AHr rechargeable, Brass coated |  |  |  |  |
|  | Working pressure | At least 0.45 Mpa |  |  |  |  |
|  | Charge input | 100-240V AC,50Hz, |  |  |  |  |
|  | Charge output | 13.5-13.8V, 2.40A |  |  |  |  |
|  | Spray flow | Not less than 2100ml\min |  |  |  |  |
|  | Time of full charge | 5 to 8 hr |  |  |  |  |
|  | Gross weight | At least 6.5 kg |  |  |  |  |
|  | Battery weight | At least 2.2 kg |  |  |  |  |
| 8 | Petrol Engine Multicrop Thresher |  |  |  |  |  |
|  | Engine | Single cylinder, Four stroke, Incline/Vertical Forced air cooled system, Oil batch type filler, Diesel Engine, Maximum power: Not less than 4 KW at rated speed, Displacement: Not less than 207cc. |  |  |  |  |
|  | Rotor Speed | Not more than 850 RPM |  |  |  |  |
|  | Roller Length | Approx. 750 mm |  |  |  |  |
|  | Rotor Diameter | Approx. 340 mm |  |  |  |  |
|  | Efficiency | Not less than 550 Kg/H |  |  |  |  |
|  | Rotor Type | Acute Teeth |  |  |  |  |
|  | Gross Weight | Approx. 80 Kg |  |  |  |  |
|  | Application | Paddy, wheat, and others |  |  |  |  |
|  | Warranty | Minimum 1 year warranty should be provided |  |  |  |  |
| 9 | Disc Mill |  |  |  |  |  |
|  | Make, Model and Brand | Any |  |  |  |  |
|  | Country of manufacturer | Any |  |  |  |  |
|  | Motor power | Not less than 7.5 Kw |  |  |  |  |
|  | Motor speed | Not less than 1450 rpm |  |  |  |  |
|  | Productivity (kg/hour) | Not less than 450 kg/hr in case of corn |  |  |  |  |
|  | Dimension (L x W x H)mm | Around 1050 x 680 x 1325 |  |  |  |  |
|  | Warranty | Minimum 1 year warranty should be provided |  |  |  |  |
| 10 | Power Tiller |  |  |  |  |  |
|  | Make, Model and Brand | Any |  |  |  |  |
|  | Country of Manufacturer | Any |  |  |  |  |
|  | Engine | Single cylinger, 4-stroke diesel engine |  |  |  |  |
|  |  | Direct combustion type |  |  |  |  |
|  |  | Specific fuel consumption around 244g/kw.h |  |  |  |  |
|  |  | Power output not less than (13.43 Kw) |  |  |  |  |
|  |  | Air cleaner oil bath type |  |  |  |  |
|  | Clutch | Multiple plates, Dry disc |  |  |  |  |
|  | Brakes | Hand operated dry brakes |  |  |  |  |
|  | No. of Gears | Forward-6, Reverse-2 (With high-low selector lever) |  |  |  |  |
|  | Tilling width | Around 900mm |  |  |  |  |
|  | Number of rotary tiller blades | Minimum 24 |  |  |  |  |
|  | Ground Clearance | Minimum 185 mm |  |  |  |  |
|  | Cooling system | Water or condense cooled |  |  |  |  |
|  | Fuel Tank Capacity | Minimum 15 liters |  |  |  |  |
|  | Lubrication System | Combined pressure and splashing type |  |  |  |  |
|  | Minimum turning radius | 1.3 meter |  |  |  |  |
|  | Starting method | Manual or Electric |  |  |  |  |
|  | Warranty | Minimum 1 year warranty should be provided |  |  |  |  |
| 11 | Diesel Mini Tiller 9 hp category |  |  |  |  |  |
|  | Make & model of machine | ISO certified, Minitiller model, Engine Model no. |  |  |  |  |
|  | Engine | 4 stroke, single cylinder, forced air cooled, Diesel engine, power: not less than 9 hp |  |  |  |  |
|  | Transmission system | Friction multi plate clutch, no. of gear speed: 2 forward & 1 reverse and neutral |  |  |  |  |
|  | Model of Transmission | Driven by gear system |  |  |  |  |
|  | Number of rotor blades | Minimum 32 blades |  |  |  |  |
|  | Transportation wheel | Pneumatic tyre |  |  |  |  |
|  | Starting method | Manual or recoil or self starting system |  |  |  |  |
|  | Weight (Kg) | Gross weight not less than 135 kg |  |  |  |  |
|  | Width of cut(cm) | 100 or above |  |  |  |  |
|  | Depth of cut (cm) | 10 or above |  |  |  |  |
|  | Fuel tank capacity (liter) | Minimum 5 liters |  |  |  |  |
|  | Warranty | Minimum 1 year warranty should be provided |  |  |  |  |
| 12 | Petrol Mini Tiller 9 hp category |  |  |  |  |  |
|  | Make & model of machine | ISO certified, Minitiller model, Engine Model no. |  |  |  |  |
|  | Engine | 4 stroke, single cylinder, forced air cooled, gasoline engine, power: not less than 9 hp |  |  |  |  |
|  | Transmission system | Friction clutch, gear system, no. of gear speed: 2 forward & 1 reverse and neutral |  |  |  |  |
|  | Model of Transmission | Driven by gear system |  |  |  |  |
|  | Number of rotor blades | Minimum 32 blades |  |  |  |  |
|  | Transportation wheel | Pneumatic tyre |  |  |  |  |
|  | Starting method | Manual or recoil starting system |  |  |  |  |
|  | Weight (Kg) | With transportation wheel: Minimum 100kg |  |  |  |  |
|  | Width of cut(cm) | 100 or above |  |  |  |  |
|  | Depth of cut (cm) | 10 or above |  |  |  |  |
|  | Fuel tank capacity (liter) | Not less than 5 liters |  |  |  |  |
|  | Warranty | Minimum 1 year warranty should be provided |  |  |  |  |